# House Upgrade Scoring Plan

A practical scoring model for prioritizing Norwegian houses/buildings for energy-grade and heating-grade improvements.

# Core Idea

Score energy performance and heating system separately because Enova describes energy grade and heating grade as independent. A building can have a weak energy grade but a good heating grade, or the opposite.

# Scores

## 1. Energy Upgrade Score

Meaning: How strongly this building should be prioritized for reducing energy use.

Suggested formula: 45% energy grade penalty + 35% energy use percentile + 15% building age score + 5% certificate age score.

Energy grade penalty: A = 0, B = 10, C = 25, D = 45, E = 65, F = 85, G = 100.

Energy use percentile should compare each building against other loaded buildings instead of using fixed universal kWh/m2 thresholds, because grade thresholds can vary by building type/category.

## 2. Heating Upgrade Score

Meaning: How strongly this building should be prioritized for improving heating source/system.

Suggested formula: 70% heating grade penalty + 20% energy use percentile + 10% building age score.

Heating penalty: Green = 0, Yellow = 35, Orange = 70, Red = 100.

Heating grade meanings: Green = very high renewable/non-electric share. Yellow = moderate renewable share. Orange = mostly electric heating. Red = predominantly direct electric or fossil-fuel heating.

## 3. Combined Retrofit Priority

Suggested formula: 55% Energy Upgrade Score + 45% Heating Upgrade Score.

Priority labels: 0-24 Low priority, 25-49 Medium priority, 50-74 High priority, 75-100 Critical priority.

# Recommendation Logic

Bad energy grade + high kWh/m2: Recommend envelope measures such as insulation, better windows/doors, ventilation improvements, and air sealing.

Bad heating grade: Recommend heating-system change, such as heat pump, district heating, bioenergy, or waterborne heating where realistic.

Bad energy grade + bad heating grade: Best full-retrofit candidate.

Good energy grade + bad heating grade: Heating-system candidate, not necessarily insulation-first.

Bad energy grade + good heating grade: Building-envelope candidate, not heating-source-first.

# Popup / App Display Idea

Each popup could show: Energy upgrade score, Heating upgrade score, Combined priority, Recommended first action, and Reason.

Example: Energy upgrade score: 82 / 100. Heating upgrade score: 71 / 100. Priority: Critical. Recommended first action: Improve building envelope + heating system. Reason: Grade G, high kWh/m2, red/orange heating score.

# Why This Helps the Pitch

This turns the project from a map that only shows energy labels into a decision-support tool that identifies the best upgrade targets.

# Sources Checked

Enova - What the energy certificate means: https://www.enova.no/energimerking/kjopeleige-bustad/hva-betyr-energimerket-for-meg/

Enova - Heating grade: https://www.enova.no/energimerking/om-energimerkeordningen/om-energiattesten/oppvarmingskarakteren/

Enova - Supported home energy measures: https://enova.no/nb/privat/bolig/tema-etterisolering

NVE - Energy efficiency: https://www.nve.no/energi/energisystem/energibruk/energieffektivisering/

DiBK - TEK17 energy minimums: https://www.dibk.no/regelverk/byggteknisk-forskrift-tek17/14/14-3

Husbanken - Upgrade loans / energy criteria: https://husbanken.no/person/lan-fra-husbanken/oppgradering/